Student Case Pack: Company-Specific Mini-Cases on Sustainable Supply Chains (Pakistan)

Instructor: Syed Rehan Ashraf | Date: 07 January 2026

# How to Use This Case Pack

Each mini-case provides a Pakistan-based company context, a brief supply chain situation, available data/assumptions, and questions you must answer. Use concepts from the lecture: Triple Bottom Line (Figure 13.1), Resource Footprint across Design–Source–Make–Deliver–Return (Figure 13.2), Transport Intensity, Eco-Efficiency, and Reverse Logistics (3Rs). Where data is missing, state assumptions clearly and show calculations.

Submission format: 3–4 pages per case, calculations + Work in groups of 2–3.

# Case 1: Interloop Ltd. – Knits/Apparel Export Sustainability

Context: Interloop is a leading hosiery/knits exporter headquartered in Faisalabad with manufacturing across Pakistan. International buyers request lower carbon and water footprints and evidence of circularity in textile waste management.

## Situation

Interloop ships finished goods weekly from Faisalabad/Lahore to Port Qasim (Karachi). Current practice uses mixed LCL shipments and ad‑hoc trucking with variable utilization (50–65%). Effluent treatment exists but Zero‑Liquid‑Discharge (ZLD) is being evaluated. Packaging is conventional (carton + polybag).

## Available Data/Assumptions

Weekly volume: 120,000 units; average unit weight: 0.25 kg; carton: 24 units.

Route: Faisalabad→Karachi ≈ 1,150 km; Lahore→Karachi ≈ 1,215 km (assume 60%/40% split).

Current truck utilization: 60%; optimized target: 85% via consolidation and pallet pattern redesign.

Fuel factor: 0.3 L/km for loaded 22–26 ft trucks; 0.22 L/km for return (lightly loaded).

Textile offcuts generated: 2.5% of fabric input; recycling partner can reclaim 80% into yarn.

## Questions (Answer with calculations)

Q1. Compute current vs optimized transport intensity (km per unit) and total diesel use for weekly shipments. What is the % reduction?

Q2. Propose a consolidation plan (LCL→FCL) and show container/truck counts by lane. Include pallet/case configuration changes.

Q3. Design a reverse‑logistics flow for textile offcuts and buyer take‑back. Estimate annual tonnes diverted from landfill and emissions avoided.

Q4. Build a resource‑footprint map (Design→Source→Make→Deliver→Return) prioritizing top 5 interventions and KPI targets for the next 12 months.

# Case 2: Nishat Mills – Spinning/Weaving/Dyeing Resource Footprint

Context: Nishat operates integrated spinning and weaving with finishing/dyeing. Global customers seek verified reductions in energy and water per kg of fabric, plus logistics network optimization to lower emissions.

## Situation

Multiple plants in Punjab supply export DCs near Karachi. Steam generation for dyeing consumes natural gas; electricity is grid + solar (10% share). Freight is mostly road with occasional rail trials.

## Available Data/Assumptions

Baseline energy: 4.5 kWh/kg fabric; baseline water: 65 L/kg fabric.

Target reductions: energy −12%, water −20% via process controls and heat recovery.

Weekly fabric output for export: 180 tonnes; average bale size: 500 kg.

Karachi port congestion adds 1.5 days average dwell; consolidation hubs may cut 20% of urban kilometers.

## Questions

Q1. Calculate energy and water savings per week if targets are met. Translate to cost if energy = PKR 45/kWh and water = PKR 0.5/L (state assumptions for tariffs).

Q2. Propose a Deliver‑stage redesign (hub‑and‑spoke + cross‑dock). Quantify km saved and emissions using your assumed factors.

Q3. Create supplier scorecards for Source‑stage (minimum 5 criteria) and a Make‑stage KPI dashboard aligned to Triple Bottom Line.

# Case 3: Engro Foods – Dairy Cold‑Chain Optimization

Context: Engro’s refrigerated distribution serves urban retailers. Diesel costs and emissions are rising; overlapping routes increase kilometers.

## Situation

Three brands run parallel routes in Lahore, each averaging 180 km/day. Trucks are 14–16 ft refrigerated vans. Micro‑hubs at city edges are being considered.

## Available Data/Assumptions

Fuel factor: 0.35 L/km for reefers.

Shared routing can remove ~35% overlaps; delivery time windows 10:00–14:00 and 17:00–20:00.

Average load utilization: 70%; target: 85% with pallet/case re‑patterning.

## Questions

Q1. Design a shared routing plan and compute daily km saved and diesel reduced. Annualize the benefit for 300 operating days.

Q2. Recommend micro‑hub locations (edge of Lahore) and quantify how time windows affect congestion and service levels.

Q3. Draft a reverse‑logistics SOP for crates and expired product returns aligned with 3Rs (reuse/recycle).

# Case 4: Unilever Pakistan – Packaging Lightweighting & Shared Distribution

Context: Unilever targets transport‑intensity reduction by lightweighting packaging and sharing backhauls with retailers. Focus corridors: Lahore–Islamabad–Peshawar.

## Available Data/Assumptions

Current packaging reduces truck cube utilization to 78%; lightweighting lifts utilization to 88% without damage risk.

Shared backhauls available with two national retailers, adding 15% more return‑load utilization.

Fuel factor: 0.28 L/km for 22–26 ft trucks.

## Questions

Q1. Quantify km and fuel savings from packaging lightweighting and shared backhauls across the corridor.

Q2. Create a Deliver‑stage network map showing DCs, cross‑dock nodes, and retailer backhaul integration; list KPIs to monitor.

Q3. Evaluate consumer‑side impacts (People lens): product damage, shelf availability, and worker safety; propose mitigation measures.

# Case 5: Daraz – Urban E‑commerce Consolidation & Returns

Context: Daraz faces rising last‑mile costs and returns management complexity in Lahore/Karachi. Congestion and fragmented courier networks lead to empty miles.

## Available Data/Assumptions

Daily orders (Lahore): 12,000; average stops per route: 40; current failed‑delivery rate: 8%.

Urban consolidation center (UCC) can reduce inner‑city runs by 20% via zone batching.

Reverse logistics: 30% of returns are non‑faulty; potential to refurbish/refurbish packaging and resell.

## Questions

Q1. Model km saved per day with UCCs and zone batching; estimate courier capacity utilization change.

Q2. Design a closed‑loop returns process (Return stage) to raise recovery rate above 70% and cut failed deliveries to 5%.

Q3. Propose a data dashboard: real‑time routing, capacity, failed deliveries, and returns recovery (KPIs + thresholds).

# Case 6: TCS & Bykea – Shared Rider Network and Micro‑Hub Collaboration

Context: Courier (TCS) and ride‑hailing logistics (Bykea) consider sharing rider capacity and setting up micro‑hubs near congested areas to reduce empty miles and improve service windows.

## Available Data/Assumptions

Average rider km/day: 95; empty‑mile share: 30%. Target: <15% with hub‑and‑spoke routing.

Delivery windows: 11:00–15:00 and 17:00–21:00; micro‑hubs reduce inner‑city traversal by 25%.

Return flows: COD reconciliation materials and small parcel returns handled at hubs.

## Questions

Q1. Compute reduction in empty miles and fuel use after micro‑hubs + shared rider dispatch.

Q2. Draft the SOP for shared distribution across competitors (data sharing, security, service levels).

Q3. Map People/Planet/Profit impacts and define 90‑day pilot metrics.

# Grading Rubric (Per Case)

(30%) Calculations & Assumptions: correctness, transparency, realism.

(25%) Design–Source–Make–Deliver–Return mapping quality and prioritization.

(20%) Feasibility & Risk analysis: operational, financial, regulatory.

(15%) People/Planet/Profit balance: measurable KPIs.

(10%) Clarity of write‑up and visuals (tables/charts/flows).

# References

Lecture book chapter (uploaded): 'Creating a Sustainable Supply Chain' (pp. 241–254). Concepts used: Triple Bottom Line (Figure 13.1), Resource Footprint (Figure 13.2), transport intensity, eco‑efficiency, reverse logistics, congestion, collaboration.